COSC 290

# Class Exercises #5

- Using the CRC polynomial 1011 (= x3 + x + 1),

  - How many bits a sender append to this information before computing CRC code word?

3

  - compute the CRC code word for the information word 1011101

1011101000 + 100 = 1011101100

  - Check the division performed at the receiver with the cord word of the result of b.

1011101100 XOR 1011 = 1000100 Remainder 000

- Using the CRC polynomial 10101 (= x4 + x3 + 1),

  - How many bits a sender append to this information before computing CRC code word?

4

  - compute the CRC code word for the information word 101101001

1011010010000 + 1001 = 1011010011001

  - Check the division performed at the receiver with the cord word of the result of b.

1011010011001 XOR 10101 = 100110111 Remainder 0000

  - If a receiver received a code word, 1011011001010112, does an error in the transmission of information? (Show your work.)

101101100101011 XOR 10101 = 10011001101 Remainder 10 implies there is an error

(work attached at bottom)

- Suppose we want an error-correcting code that will allow all single-bit errors to be corrected for memory words of length 10.

  - How many parity bits are necessary?

4

  - Assuming we are using the Hamming algorithm presented in this chapter to design our error-correcting code and an odd parity , find the code word to present the 10-bit information word:

1   0  0  1  1  0  0  1  1  1

10011000110100 to have odd parity across all 4 parity groups

  - We now receive the following code word:

1  0  1  0  1  1  0  0  0  0  1  0  1  1

Is this a legal code word, assuming an odd parity?  If not, what is the corrected code word? (Which bit is the error bit?)

No, this is not a legal code word, there is an error in bit 9. The corrected code word is

10101000001011

WORK